RESEARCH PROPOSAL FORM

(also referred to as the ‘Statement of Intent Form’, or SOI)

To be submitted by the researcher to the Institute Research Sub-Committee (IRC)

| Research Title: Computer Vision Based Calibration and Verification System for Physical Roulette Wheels |  |
| --- | --- |
| Institute name Institute of Information & Communication Technology |  |
| Course / Programme: Bachelor of Science (Honours) in Software Development |  |
| Level and year of study Level 6 – Year 2 |  |
| Main area of study being proposed: Modern quality-assurance systems combine computer vision and statistical quality control to calibrate and verify mechanical precision in automated devices. Unlike electronic RNGs regulated under GLI-11, physical roulette wheels lack equivalent calibration verification tools despite being governed by the same fairness principles. This project develops a computer-vision-based calibration and verification system that automatically logs outcomes, models wheel geometry, and analyses pocket consistency over time. The system focuses on verifying that the wheel’s mechanical alignment and operational performance remain consistent with theoretical fairness assumptions. |  |
| Name of Researcher: Gabriel Young | Researcher’s I.D. Number: 14705L |
| Signature of Researcher | Date of submission of Form |
| Name of Tutor (or Recommended Tutor): |  |

Personal Motivation for the Choice of Research Theme.

Enter details here: Roulette wheels operate as expected under fairness assumptions yet mechanical wear in the machines produces outcome biases which endanger consumer protection. The creation of a budget-friendly automatic calibration tool fulfils my interest regarding randomness boundaries and simultaneously provides practical benefits to casinos and regulatory bodies as well as gaming participants. Manual logging of roulette outcomes is trivial but impractical for the hundreds of thousands of spins needed to detect subtle biases. Automating this process through computer vision is both technically challenging and crucial for practical fairness calibration.

Outline of Key Literature and Theoretical Framework or Propositions.

Enter details here: 1. Computer Vision Foundations for Roulette Ball Detection This project needs high-frame-rate visual tracking of the roulette ball and wheel since all subsequent statistical tests require an accurate time-stamped sequence of observed outcomes. The YOLO family of single-shot convolutional networks has proven capable of precise localization of small fast-moving targets within dynamic cluttered environments at high frame rates (e.g., >30 frames per second) using consumer-grade GPUs. While identifying a roulette result manually is trivial, automating this process with high-speed, small-object detection under challenging casino conditions is not. The literature on YOLO-based small-object tracking [1], [2], [3] shows the non-trivial technical challenges that my work builds on. Traffic-safety prototypes: Attard employed YOLOv5 with COCO pre-training to develop adaptive traffic controllers which accurately detected vehicles and pedestrians [1]. The YOLO family of detectors proved to maintain its robust performance after receiving fine-tuning from relatively small domain-specific datasets according to the research findings. The research by Attard focuses on traffic management but his investigation about input resolution and anchor box sizes delivers immediate applications for roulette video object detection since the ball occupies fewer than 40 pixels in each frame. In-cab distraction monitoring: Cassar utilized YOLOv3-320 and MediaPipe landmarks for driver distraction monitoring to detect mobile phone usage with 83% precision and 90% recall under different lighting conditions [2]. The two-stage cascade approach developed by Cassar includes a first coarse detector followed by a second finer detector which provides a beneficial architectural model for roulette analysis. The first detector should identify the entire roulette wheel as a region of interest (ROI) before a specialized detector locates the ball within that area. Small-object vehicle tracking: The YOLOv8 model successfully demonstrated its ability to detect vehicles for roadside speed sensing according to Shaqib et al. who reported a mean absolute error of 3.5 km/h despite facing occlusion and motion blur challenges [3]. The authors implemented data augmentation procedures that added Gaussian noise along with motion blur and random brightness variations to their dataset. The roulette ball detector would gain real-world casino environmental resistance through this approach because it would be able to withstand variable lighting conditions and roulette wheel surface reflections. The fundamental computer vision research establishes that a specifically trained YOLO variant using labelled roulette frame data will produce precise outcome sequences with timestamp information which supports advanced randomness assessments. 2. Statistical Tests for Uniformity and Independence After collecting precise visual outcome data, the main project function becomes statistical analysis to assess if observed roulette outcomes differ significantly from perfect independent identically distributed (i.i.d.) uniform behaviour. Although GLI-11 and similar standards define statistical tests for electronic randomness, these do not apply to physical roulette wheels. My project applies the same rigor to the mechanical domain, where no scalable auditing tool currently exists. Classical Chi-Squared (χ²) goodness-of-fit: Sani's "Comparative Analysis of Random Number Generators Using Monte-Carlo Algorithm" examines both the Chi-squared frequency test (goodness-of-fit) and the Chi-squared independence test (contingency tables) as described in his thesis [4]. The thesis includes available open-source code for these implementations. This project will use the Chi-squared frequency test and Chi-squared independence test as its statistical foundation because they serve as standard first-line diagnostics in gaming applications. The results obtained by Sani in his research show that generators passed uniformity tests but failed independence tests thus demonstrating the necessity of using multiple statistical tests for thorough evaluation. Regulatory standards: The GLI-11 standard presents RNG requirements for electronic gaming devices in Chapter 3 according to the standard. The Chi-squared "Total Distribution" test must run at 99% confidence level according to [9] along with serial-correlation tests on at least 10 million outcomes. Physical roulette wheels generally fall outside GLI-11 scope yet European regulators now use these strict statistical standards as industry best practices to evaluate mechanical randomness. This project will base its testing approaches on the rigorous industry standards to guarantee both robustness and credibility of fairness evaluations. 3. Sample-Size and Power Considerations Detecting subtle biases in roulette outcomes, such as a 2-3% deviation in a single pocket's probability, with adequate statistical power (e.g., 0.8 power at an alpha level of 0.01) necessitates a substantial volume of data, potentially upwards of 500,000 spins. Two complementary sources quantify this demanding data requirement: Rodríguez & Mendes: The authors Rodríguez and Mendes in their book "Probability, Decisions and Games" explain through Chebyshev bounds and Monte Carlo simulations that statistical significance for detecting a 5% deviation in single pocket outcome frequencies requires more than 300,000 trials [7]. Their work shows that empirical frequency matches true probability when running numerous flips thus supporting the need for extensive data collection in randomness testing. Newsom's lecture notes: According to Newsom's lecture notes on sample size analysis for categorical data the Chi-squared test with one degree of freedom needs 800 observations to detect a small effect size (Cohen's w ≈ 0.1) [8]. The requirement increases to more than 500,000 observations to detect minor biases when maintaining casino-level alpha significance. These substantial data requirements dictate that as much data is possible should be collected, followed by an automated, object-detection-driven transcription process to convert video frames into a quantifiable sequence of outcomes. 4. Empirical Evidence of Physical and Mechanical Bias Numerous research studies show that real roulette wheels deviate from perfect uniformity according to statistical models and theoretical tests because they display actual mechanical and physical biases. Small & Tse (2012): Small and Tse demonstrated a notable "first-pocket" bias using high-speed video analysis together with differential-equation modelling. The researchers observed that a 0.2° table tilt produced an 18% expected-return advantage through controlled laboratory experiments, yet this deviated from the theoretical 2.7% randomness benefit [6]. The study shows that manufacturing defects along with environmental conditions create exploitable non-randomness in systems. Abuhanna (2015): Abuhanna monitored thousands of casino roulette spins to confirm that the outcome distribution was not random. The observed bias stemmed from how the ball started its roll which proved that roulette operated outside the traditional chance-based model [5]. Abuhanna applied statistical methods to prove that wheel deformities create biased outcomes between strong and weak numbers which do not match statistical expectations. The practical application of strict fairness tests to physical roulette systems receives validation from these significant case studies yet demonstrates why continuous monitoring of roulette wheel behaviour should replace one-off certifications. The continuous assessment process ensures both fairness and integrity of the game throughout time. 5. Manufacturing Precision and Calibration Standards Physical roulette wheels are manufactured under strict mechanical tolerances and are subject to ongoing certification by national gaming authorities. These regulatory documents emphasise the importance of unbiased design, precise manufacturing, and periodic inspection to guarantee fairness. Liquor and Gaming Authority of Manitoba’s Gaming Integrity Standards - Roulette Wheels specify explicit construction and calibration requirements, including minimum wheel and cylinder diameters, the equal distribution of weight across the rotor, levelness between wheel and bowl, and verification that all pockets are identical in depth and spacing [10]. The standard further requires that every roulette wheel be balanced and levelled by the manufacturer prior to shipment and that quality-assurance documentation accompanies each unit to confirm compliance [10]. pillemyndigheden Testing Standards for Land-based Casinos in Denmark require that each roulette wheel undergo accredited testing by certified laboratories and that certification be renewed every two years [11]. These guidelines reinforce that mechanical randomness and calibration stability are continuously monitored within regulated jurisdictions. Australian / New Zealand Gaming Machine National Standard (Rev 11) also outlines stringent calibration, conformity, and inspection requirements for gambling equipment to ensure that all mechanical and electronic devices operate within predefined fairness thresholds [12]. This demonstrates that roulette-wheel integrity is already governed by rigorous standards, but current methods depend on manual inspection and physical measurement tools. Despite these existing standards, calibration and verification remain labour-intensive and visually assessed. This gap highlights an opportunity for computer-vision-based calibration systems that automate mechanical verification, quantify geometric consistency, and detect potential misalignment earlier and more objectively than current manual procedures.

Significance of the Study.

Enter details here: Efficient, low cost, system for 1) calibrating a wheel 2) constant monitoring of the wheel operations over time The significance of this study lies in introducing an automated calibration and verification framework for physical roulette wheels. Electronic RNGs have well-defined validation standards such as GLI-11, yet equivalent calibration tools for physical wheels are lacking. By combining computer vision based outcome logging, homography driven geometric mapping, and statistical consistency checks, this project provides a low-cost, reproducible method to verify whether a wheel continues to operate within its designed mechanical tolerances. The system supports manufacturers, regulators, and operators in maintaining fairness assurance without requiring long manual observation sessions or specialized proprietary equipment.

Hypotheses and/or Research Question/s

Enter details here: Research Hypothesis: A computer-vision-based calibration and verification system can reliably measure the mechanical consistency of roulette wheels by detecting deviations in geometric alignment or pocket occupancy distributions, confirming whether the wheel remains within fair operational tolerance. Research Questions: - What computer-vision methods (e.g., YOLO detection, homography) most effectively support automated geometric mapping and pocket identification in physical roulette wheels? - How accurately can the system measure mechanical or geometric deviations that might affect fairness assumptions? - Can the proposed calibration tool quantify and visualize wheel consistency over repeated trials in a way that complements traditional mechanical inspection methods?

Target Participants and Research Methods for Data Collection and Analysis

Enter details here: The core methodology involves developing a computer-vision calibration prototype using YOLO (You Only Look Once) for ball and wheel detection. The system’s main function is to map wheel geometry and verify mechanical consistency through automated visual tracking of the ball and its final landing pocket. The system will make use of a geometric homography calibration technique. Each roulette wheel frame is analysed to locate the outer and inner rims, from these, a projective transformation rectifies the tilted camera view to a top-down circular reference. The green “0” pocket is detected using colour segmentation and establishes the angular origin for pocket indexing. Subsequent pockets are then mapped mathematically according to the standard European wheel sequence. YOLO model training data will be collected from publicly accessible roulette gameplay videos. The videos will be divided into frames and undergo detailed annotation for ball position identification and pocket boundary definition. The labelling process becomes faster by importing pre-trained object detection models from Roboflow platforms which users review and correct before merging with hand-annotated data. The proposed method combines pre-existing datasets from multiple platforms to generate an extensive annotation base with multiple camera perspectives under various lighting settings which reduces the total labelling effort. The prototype will undergo performance evaluations at two critical stages. Detection evaluation, precision, recall, and F1-score will measure the reliability of the YOLO-based ball and wheel detectors, with comparisons made against existing roulette datasets. Calibration evaluation, once detections are consistent, the system’s calibration accuracy will be tested by examining how well it detects intentional biases in controlled datasets. Using spins recorded under baseline, tilted, and weighted conditions, the analysis will determine whether the system correctly identifies measurable deviations in pocket distributions and wheel geometry. Two complementary data sources will be employed. Public roulette footage will demonstrate the system’s feasibility and detection accuracy under diverse visual conditions. A consumer-grade physical roulette wheel will provide controlled experimental data for calibration analysis. Baseline spins will be recorded on a level surface, followed by deliberate, reversible perturbations such as minor table tilt (≈ 0.5°–2°) or the addition of small counterweights beneath the rim. Each configuration will yield an independent dataset for comparison. Although Rodríguez & Mendes [7] and Newsom [8] show that detecting extremely small probabilistic biases demands hundreds of thousands of trials, this study’s objective is different, it focuses on identifying mechanical and geometric deviations introduced by calibration changes. Drawing on their statistical reasoning, approximately 200–500 recorded spins per condition (baseline, tilt, weighted) will provide adequate power to reveal distributional shifts while remaining feasible within the project’s timeframe. Comparative analyses, such as χ² frequency tests and effect-size estimation, will quantify how wheel imperfections influence observed outcomes. The calibration tool will process the results gathered from the computer-vision component and apply statistical methods described in the literature review, such as Chi-squared goodness-of-fit and independence tests. These analyses will generate clear calibration outputs, including: - Pocket-frequency tables comparing how often each pocket appears under different conditions - Visual distribution or polar plots showing angular regions where results cluster - Statistical indicators showing the magnitude of deviation from the baseline. By comparing these outputs across baseline and manipulated experiments, the study will evaluate the system’s ability to detect physical misalignment or imbalance in a roulette wheel.

Anticipated Contributions of the Study.

Enter details here: This research project delivers an innovative automated system which regulatory organizations and the gaming industry can use to conduct unbiased roulette wheel assessments. The research addresses the existing knowledge gap by providing ongoing independent verification of mechanical randomness in live gaming environments through an affordable scalable calibration tool despite previous research showing physical wheel biases. The research outcomes will improve consumer protection measures while improving the probability models used in gaming.

Dissertation Project Plan.

Enter details here: • Jul 2025 – Refine draft proposal & expand literature review - in the meantime look for more related research • Aug 2025 – Gather sample roulette videos from different sources and set up annotation tools • Sep 2025 – Finalise data gathering and begin labelling • Oct 2025 – Finalise and submit project proposal • Nov 2025 – Finish labelling -> Train first YOLO detector • Dec 2025 – Add video processing code -> go through videos of continuous spins and log outcomes • Jan 2026 – Conduct initial statistical tests on logged outcomes. • Feb 2026 – Analyse statistical results and compare against theoretical models. • Mar 2026 – Start finalising written paper (should have been working on it throughout – with feedback from supervisor) • Apr 2026 – Complete full dissertation draft. If time permits, possibly look for a proofreader • May 2026 – Final edits, submission of dissertation & prototype. PRESENT VIVA

Ethical Considerations. Refer to guidance points below. You are also additionally required to read MCAST Document 074 ‘Research Ethics Policy and Procedure’ that is available on the College website Research shall be conducted in such a manner so as to avoid any psychological and physical harm to humans and animals and financial damage to organizations: - Only the supervisor and examiners will have access to any data gathered. - Participants will remain free to withdraw from the study at any time without having to provide any reason. In the case of withdrawal, all the records and information collection will be deleted. - The participant, who is the sole proprietor of the data provided, is granting that such data would be processed for this study purposes only. - The data collection process will be a transparent process. - All transcriptions and/or electronic recordings reflecting the data collected, once exhausted, are to be deleted - Confidentiality, anonymity and data protection procedures are to be ethically abided by. - The researcher would provide a soft copy of the study to the participant, if required.

Enter details here regarding possibility of issues regarding confidential personal data: How will you ensure that: - No personal data or confidential data is divulged. - Participants’ identities are not divulged (ie kept anonymous) Publicly available roulette gameplay videos will be the exclusive source of information for data collection. The data collection process will not acquire or save any personal information or identifying details that exist in the video content. When processing videos, researchers will apply automatic blurring or masking techniques on visible personal data to prevent exposure of personal information (e.g., background faces). The video sources will receive numerical codes instead of their original names to maintain privacy protection. The research team will store all raw video footage along with extracted data on encrypted local drives that can only be accessed by the researcher and approved academic personnel (supervisor, examiners) until the study completion and successful submission when the data will be permanently deleted.

Enter details here regarding possibility of physical harm: - How will you ensure that no person or animal gets hurt during the implementation of the research? - What Personal Protective Equipment (PPE) will you be needing/using? The research focuses on conducting computational analyses of pre-recorded roulette gameplay videos that exist in the public domain. The study will operate without requiring human subject participation or animal involvement or physical casino settings or equipment throughout its execution. The research design does not contain any elements that would lead to physical injuries for humans or animals. The research requires no Personal Protective Equipment (PPE) since all activities occur within a safe virtual academic space.

Enter details here regarding possibility of moral harm: - What steps will you take to avoid unduly offending or disturbing the well-being of the participants? - How will you avoid any possible psychological, spiritual or cultural offence to participants? - How will you ensure that the interests and rights of minors / vulnerable / disabled persons / specific social groups are safeguarded where necessary? The research methodology excludes any direct human contact along with surveys and interviews. The collected data consists of roulette gameplay videos which are publicly accessible. Since there is no direct involvement of participants in data collection activities, the study poses no risk of causing offense or disturbing well-being or psychological or spiritual or cultural offense to participants. The research investigates mechanical and statistical aspects of game results instead of focusing on individual players. The research does not involve working with minors, vulnerable people or disabled persons or social groups that require additional safeguards beyond general data anonymization measures.

Enter details here regarding possibility of business harm: How will you ensure that: - Participants do not suffer any competitive disadvantage as an outcome of the research? - Confidential business ideas and data are protected and not divulged? The research will exclusively depend on roulette gameplay videos available to the public along with open-source statistical analysis methods. The research will not require any proprietary or confidential business data from casinos or gaming organizations since it will not need access to or disclosure of their information. The research examines generic roulette mechanics and statistical results to demonstrate fairness rather than seeking to reveal or exploit operational vulnerabilities from specific businesses. The analysis will focus on statistical observations about randomness and aims to enhance gaming integrity through consumer trust promotion rather than creating competitive disadvantages for businesses.

Please see Annex 1 for a sample Participant Information Letter and Annex 2 for a sample Participant Consent Form. Student is to submit a copy of the proposed Participant Information Letter and Participant Consent Form where applicable. Both documents should be attached to the end of the SOI that is being submitted by the student.

List of Key References:

[1] G. Attard, “Modernizing the traffic light system to reduce waiting time,” B.Sc. (Hons.) Software Development, MCAST Institute of Information and Communication Technology, 2023. [2] S. Cassar, “Detecting Driver Distractions through Computer Vision,” B.Sc. (Hons.) Software Development, MCAST Institute of Information and Communication Technology, 2023. [3] S. Shaqib, A. P. Alo, R. S. Sultan, U. Haque, S. S. Khan, and M. S. Rahman, “Vehicle Speed Detection System Utilizing YOLOv8: Enhancing Road Safety and Traffic Management for Metropolitan Areas,” arXiv preprint arXiv:2406.07710, 2024. [4] Y. A. Sani, “Comparative Analysis of Random Number Generators Using Monte Carlo Algorithm,” B.Sc. thesis, Dept. Computer Science, Yobe State Univ., Damaturu, Nigeria, Nov. 2017. [Online]. Available: https://www.researchgate.net/publication/391903530_Comparative_Analysis_of_Random_Number_Generators_Using_Monte_Carlo_Algorithm [5] D. Abuhanna, “Roulette: More than just a Chance,” B.S. honors thesis, Univ. of Nevada, Las Vegas, NV, USA, 2015. [Online]. Available: https://digitalscholarship.unlv.edu/honors_theses/27 [6] M. Small and C. K. Tse, “Predicting the outcome of roulette,” Chaos, vol. 22, no. 3, p. 033150, Sep. 2012, doi: 10.1063/1.4753920. [Online]. Available: https://arxiv.org/abs/1204.6412 [7] A. Rodríguez and B. Mendes, Probability, Decisions and Games: A Gentle Introduction using R. Hoboken, NJ, USA: Wiley, 2018, ch. 3. [Online]. Available: https://www.programmer-books.com/wp-content/uploads/2018/08/probability-decisions-games.pdf [8] J. F. Newsom, “Sample Size Issues for Categorical Analyses and Logistic Regression,” lecture notes, Portland State Univ., Portland, OR, USA, 2021. [Online]. Available: https://web.pdx.edu/~newsomj/cdaclass/ho_sample%20size.pdf [9] Gaming Laboratories International, GLI-11: Standards for Gaming Devices in Casinos, ver. 3.0, Sep. 2016. [Online]. Available: https://gaminglabs.com/wp-content/uploads/2018/09/GLI-11-Gaming-Devices-V3-0.pdf [10] Liquor and Gaming Authority of Manitoba. Gaming Integrity Standards: Roulette Wheels. Manitoba Liquor and Lotteries Corporation, 2018. [Online]. Available: https://www.tistandards.ca/pdf/roulette_wheel_standards.pdf [11] Spillemyndigheden. Testing Standards for Land-based Casino (SCP.01.05.EN.1.1). Danish Gambling Authority, 2020. [Online]. Available: https://www.spillemyndigheden.dk/uploads/2020-09/SCP.01.05.EN_.1.1 - Testing Standards for Land-based Casino - WT.pdf [12] Commonwealth of Australia & New Zealand Government. Australian / New Zealand Gaming Machine National Standard (Revision 11). 2018. [Online]. Available: https://www.cbs.sa.gov.au/documents/gaming-machine-national-standards.pdf

This section is to be filled in by the representative of the Institute Research Sub-Committee (IRC) prior to forwarding of this Form to the ‘MCAST Research Ethics Committee’ for final ethics approval:

| Nature of Ethical Consideration | Outcome (Tick) | Comments/Advice |
| --- | --- | --- |
| All ethical issues have been adequately tackled. |  |  |
| Possibility of issues regarding misuse of data or some form of harm. |  |  |

| Details of Representative to the Institute Research Sub-Committee. |  |
| --- | --- |
| Name | Signature |
| Designation | Date |

Annex 1:  Participant Information Letter

Sample:

Title of Research:   ___________________________________________________________________

You are being invited to take part in a research study.  Before you decide to participate, it is important for you to understand why the research is being done and what it will involve.  Please take time to read the following information carefully and discuss it with others if you wish.  Ask us if there is anything that is not clear or if you would like more information.

What is the purpose of the study?

This research is being undertaken on…

Why have I been chosen?

You have been chosen because…

Do I have to take part?

It is up to you to decide whether or not your take part.  If you decide to take part you will be given this information sheet to keep and be asked to sign a corresponding consent form.

What will happen to me if I take part?

You will then be given a questionnaire on.../your data will be used…/your image will be used…

What are the possible disadvantages and risks of taking part?

There are no disadvantages or risks foreseen in taking part in the study.

What are the possible benefits of taking part?

By taking part you will be contributing to the development of a set of recommendations for…

What if something goes wrong?

If you wish to complain or have any concerns about any aspect of the way in which you have been approached or treated during the course of this study, please contact…(researcher is to give his/her MCAST email as a contact)

Will my details be kept confidential?

All information which is collected about you during the course of the research will be kept strictly confidential so that only the researcher carrying out the research will have access to such information and will not be shared with any other individuals. Participants should note that data/images collected from this project may be retained and published in an anonymized form. By agreeing to participate in this project, you are consenting to the retention and publication of data.

What will happen to the results of the research study?

The results will be written up into a dissertation for my final project of my Bachelor…

Who is organizing the research?

The research is conducted as part of a degree in …

Who may I contact for further information?

If you would like more information about the research before you decide…(researcher is to give his/her MCAST email as a contact)

Thank you for your interest in this research…

Annex 2:  Participant (or Guardian) Consent Form

Sample:

Title of Research: ____________________________

Name of Researcher:  ___________________________

# Please initial box

- I confirm that I have read and understand the Information Letter

for the above study and have had the opportunity to ask questions.

- I understand that my/my charge’s participation is voluntary and that I/my charge am/are free to withdraw at any time without giving any reason.

3.	I agree to allow my daughter/son/charge to take part in the above study.

(Statement 3 is to be included only when guardians/parents are involved in giving consent)

___________________                _________________  	_____________

Name of Participant/

Guardian	Date		Signature

Researcher	Date	Signature

1 for participant; 1 for researcher